- Category 7 cable

- From Wikipedia, the free encyclopedia

Category 7 cable (CAT7), (ISO/IEC 11801:2002 category 7/class F), is a cable standard for Ethernet and other interconnect technologies that can be made to be backwards compatible with traditional CAT5 and CAT6 Ethernet cable. CAT7 features even more stringent specifications for crosstalk and system noise than CAT6. To achieve this, shielding has been added for individual wire pairs and the cable as a whole.

The CAT7 cable standard has been created to allow 10 gigabit Ethernet over 100 meters of copper cabling. The cable contains four twisted copper wire pairs, just like the earlier standards. CAT7 can be terminated either with RJ-45 compatible GG45 electrical connectors which incorporate the RJ-45 standard or with TERA connectors. When combined with GG-45 or TERA connectors, CAT7 cable is rated for transmission frequencies of up to 600 MHz.

Unshielded & Shielded Twisted Pair Cabling Standards v • d • e

- Cat 1: Currently unrecognized by TIA/EIA. Previously used for POTS telephone communications, ISDN and doorbell wiring. - Cat 2: Currently unrecognized by TIA/EIA. Previously was frequently used on 4 Mbit/s token ring networks. - Cat 3: Currently defined in TIA/EIA-568-B, used for data networks using frequencies up to 16 MHz. Historically popular for 10 Mbit/s Ethernet networks. - Cat 4: Currently unrecognized by TIA/EIA. Provided performance of up to 20 MHz, and was frequently used on 16 Mbit/s token ring networks. - Cat 5: Currently unrecognized by TIA/EIA. Provided performance of up to 100 MHz, and was frequently used on 100 Mbit/s ethernet networks. May be unsuitable for 1000BASE-T gigabit ethernet. - Cat 5e: Currently defined in TIA/EIA-568-B. Provides performance of up to 100 MHz, and is frequently used for both 100 Mbit/s and gigabit ethernet networks. - Cat 6: Currently defined in TIA/EIA-568-B. It provides performance of up to 250 MHz, more than double category 5 and 5e. - Cat 6a: Future specification for 10 Gbit/s applications. - Cat 7: An informal name applied to ISO/IEC 11801 Class F cabling. This standard specifies four individually-shielded pairs (STP) inside an overall shield. Designed for transmission at frequencies up to 600 MHz.